- Environment requirement

Ruijie Auto Discovery VPN (ADVPN) allows to dynamically establish direct tunnels (called shortcuts) between the spokes of a traditional Hub and Spoke architecture.
After a shortcut tunnel is established between two spokes and routing has converged, spoke to spoke traffic no longer needs to flow through the Hub. Direct connectivity is provided.

- Topology

- Configure point

（1）configure HUB

1）the based internet configuration

2）phase1 configuration

3）phase2 configuration

4）configure IPsec VPN tunnel IP

5）configure BGP and Route-Reflector

6）configure policy

(2) configure SPOKE1

1）the based internet configuration

2）phase1 configuration

3）phase2 configuration

4）configure IPsec VPN tunnel IP

5）configure BGP and Route-Reflector

6）configure policy

(3) configure SPOKE2

1）the based internet configuration

2）phase1 configuration

3）phase2 configuration

4）configure IPsec VPN tunnel IP

5）configure BGP and Route-Reflector

6）configure policy

- Configure step

- configure HUB

1)the based internet configuration

wan1:  202.1.1.1/24

internal:  172.16.1.0/24

the detail configuration refer to this guide

https://www.ruijienetworks.com/support/documents/slide_59169

2)configure phase 1(CLI)

Athu way：pre-shared key

config vpn ipsec phase1-interface

edit "ADVPN"

set type dynamic

set interface "wan1"

set psksecret 111111

set add-route disable

set auto-discovery-sender enable

end

- because Hub need to connect with different spoke(static IP /PPPOE) using IPsec VPN, so HUB need to choose dynamic mode

- ADVPN don’t support aggressive mode，only can choose main mode

- HUB need to enable “auto-discovery-sender enable”，so that it can receive and send the data between Spokes.（Hub response information，let Spoke know what public ip tunnel you should connect to）

- Because used BGP，so need to close auto add route function “add-route disable”

3) configure phash2(CLI)

config vpn ipsec phase2-interface

edit "ADVPN"

set phase1name "ADVPN"

next

end

4)configure IPsec VPN tunnel IP

config system interface

edit "ADVPN"

set vdom "root"

set ip 10.10.10.1 255.255.255.255

set allowaccess ping

set type tunnel

set remote-ip 10.10.10.254       （10.10.10.254 isn’t a actual IP）

set interface "wan1"

next

end

5)configure BGP and route-reflector

config router bgp

set as 100

set router-id 10.10.10.1

config neighbor-group                                  //  -neighbor-group feature

edit "ADVPN-PEERS"

set remote-as 100

set route-reflector-client enable             //-configure route-reflector

next

end

config neighbor-range

edit 1

set prefix 10.10.10.0 255.255.255.0

set neighbor-group "ADVPN-PEERS"

next

end

config network

edit 1

set prefix 172.16.1.0 255.255.255.0          // -release route

next

end

Notice：

1.use BGP neighbor-group feature，BGP-Peer only need to match prefix list 10.10.10.0/24, and then can build BGP neighbor with HUB.

2.configure neighbor to Route-Reflector-Client，itself will be Route-Reflector, and then it can sync total route through route-reflector feature.

6) configure policy

config firewall policy

edit 1

set srcintf "internal"

set dstintf "ADVPN"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

edit 2

set srcintf "ADVPN"

set dstintf "internal"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

edit 3

set srcintf "ADVPN"

set dstintf "ADVPN"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

end

- configure SPOKE1

1)based network configuration

1. The same with Hub

2. Interface IP configure：

wan1:  202.2.2.1/24

internal:  172.16.2.0/24

2)configure phase1(CLI)

config vpn ipsec phase1-interface

edit "ADVPN"

set interface "wan1"

set remote-gw 202.1.1.1

set psksecret 111111

set add-route disable

set auto-discovery-receiver enable

next

end

Notice：

a. because HUB public IP is static，so Spoke need to configure static IPsec VPN

b. Spoke need to enable “auto-discovery-receiver enable”，so that it can receive the ADVPN data of Hub.

c. because Spoke need to use BGP，so need to disable add-route function（add-route disable）

3) configure phase 2(CLI)

config vpn ipsec phase2-interface

edit "ADVPN"

set phase1name "ADVPN"

set auto-negotiate enable

next

4) configure IPsec VPN tunnel IP

config system interface

edit "ADVPN"

set vdom "root"

set ip 10.10.10.2 255.255.255.255

set allowaccess ping

set type tunnel

set remote-ip 10.10.10.1      // the tunnel IP point to Hub

set interface "wan1"

next

end

5)configure BGP and route-rejector

config router bgp

set as 100

set router-id 10.10.10.2

config neighbor

edit "10.10.10.1"                       \\ only need to build BGP neighbor with Hub

set remote-as 100

next

end

config network

edit 1

set prefix 172.16.2.0 255.255.255.0

next

end

configure static route to let IBGP up

config router static

edit 2

set dst 10.10.10.0 255.255.255.0

set device "ADVPN"

next

end

Notice：

All Spoke need to configure 10.10.10.0/24 next-hop point to ADVPN，the route of BGP is based on IGP，So need to make sure IGP route can be up，or will have route hole。（because SPOKE1 learned the BGP route 172.16.3.0/24 of SPOKE2 next-hop is point to 10.10.10.3，but SPOKE1 go to 10.10.10.3 using static route，will be forward to ISP device，the forwarding business data between SPOKE will be wrong，at the same time can’t touch off ADVPN，So need to add a static route to 10.10.10.0/24 point to ADVPN tunnel.

6) configure policy

config firewall policy

edit 0

set srcintf "internal"

set dstintf "ADVPN"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

edit 0

set srcintf "ADVPN"

set dstintf "internal"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

end

- configure SPOKE2

1)based network configuration

The same with Hub

Interface configure

wan1:  202.3.3.1/24

internal:  172.16.3.0/24

2)configure phase 1(CLI)

config vpn ipsec phase1-interface

edit "ADVPN"

set interface "wan1"

set remote-gw 202.1.1.1

set psksecret 111111

set add-route disable

set auto-discovery-receiver enable

next

end

3)configure phase 2(CLI)

config vpn ipsec phase2-interface

edit "ADVPN"

set phase1name "ADVPN"

set auto-negotiate enable

next

4)configure IPsec VPN tunnel IP

config system interface

edit "ADVPN"

set vdom "root"

set ip 10.10.10.3 255.255.255.255

set allowaccess ping

set type tunnel

set remote-ip 10.10.10.1      // the tunnel IP point to HUB

set interface "wan1"

next

end

5)configure BGP and route-rejector

config router bgp

set as 100

set router-id 10.10.10.3

config neighbor

edit "10.10.10.1"

set remote-as 100

next

end

config network

edit 1

set prefix 172.16.3.0 255.255.255.0

next

end

configure static route to let IBGP be up打通IBGP

config router static

edit 0

set dst 10.10.10.0 255.255.255.0

set device "ADVPN"

next

end

6) configure policy

config firewall policy

edit 0

set srcintf lan

set dstintf "ADVPN"

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enablb

next

edit 0

set srcintf "ADVPN"

set dstintf  lan

set srcaddr "all"

set dstaddr "all"

set action accept

set schedule "always"

set service "ALL"

set status enable

next

end

- check configuration result

VPN->Monitor->Ipsec monitor

Route -> Monitor-> routing Monitor