4G Router backup link configuration guide

Diagram

Configuration Steps

Ruijie#conf t

Ruijie(config)#int fastEthernet 1/0

Ruijie(config-if-FastEthernet 1/0)#switchport mode access

Ruijie(config-if-FastEthernet 1/0)#switchport access vlan 1

Ruijie(config)#int vlan 1

Ruijie(config-if-VLAN 1)#ip add pulbic-address mask  // Configure wired ip address

Ruijie#sh ip interface brief   // Check whether cellular interface has got ip address.

Noted: this guide is based on IP address has been configured and connectivity is fine.

- Configure RNS detection

Ruijie#conf t

Ruijie(config)#ip rns 10

// Detect wired interface . Need to configure next-hop

Ruijie(config-ip-rns)#icmp-echo 50.7.154.1 out-interface vlan 1 next-hop 50.7.154.1

//Assume 50.7.154.1 is gateway of wired public address

Ruijie(config-ip-rns-icmp-echo)#timeout 3000

// if receive icmp echo after 3s, determine the packet timeout

Ruijie(config-ip-rns-icmp-echo)#frequency 3000

//Send detection every 3s

Ruijie(config-ip-rns-icmp-echo)#ntime 3

//If the timeout is exceeded for 3 times, the rns check fails

Ruijie(config)#track 10 rns 10

Ruijie(config-track)#delay down 3 up 3

//If the rns detection fails, the track status is delayed for 3s and then becomes down. If no delay is configured, the track status will change immediately.

Ruijie(config)#ip rns 20

// Detect cellular interface

Ruijie(config-ip-rns)#icmp-echo 8.8.8.8 out-interface cellular 0/0

Ruijie(config-ip-rns-icmp-echo)#timeout 3000

Ruijie(config-ip-rns-icmp-echo)#frequency 3000

Ruijie(config-ip-rns-icmp-echo)#ntime 3000

Ruijie(config)#track 20 rns 20

Ruijie(config-track)#delay down 3 up 3

- Configure floating static route and binding track

Ruijie(config)#ip route 0.0.0.0 0.0.0.0 vlan 1 50.7.154.1 track 10

//primary

Ruijie(config)#ip route 0.0.0.0 0.0.0.0 cellular 0/0 100

//backup

- Enable interface linkage function with wired on 4G Interface

Ruijie(config)#interface cellular 0/0

Ruijie(config-if-Cellular 0/0)#apply detect interface vlan 1 track 10

//auto-dial when wired interface is down. 1 is wired interface vlan

Ruijie(config-if-Cellular 0/0)#apply dial-on-demand

Ruijie(config-if-Cellular 0/0)#profile create master track 20

Ruijie(config)#svi-interface detecting

//enable SVI detecting

- If IPsec has configured on 4G cellular, this command is needed. In order to dismantle IPsec tunnel when wired interface is up again.

Ruijie(config)#crypto isakmp link-redundency backup cellular 0/0 track 10

- Enable NAT feature

Router(config)#access-list 1 permit any

Ruijie(config)#ip nat pool test prefix-length 24

Ruijie(config-ipnat-pool)#address interface VLAN 1 match interface VLAN 1

Ruijie(config-ipnat-pool)#address interface Cellular 0/0 match interface Cellular 0/0

Ruijie(config)# ip nat inside source list 1 pool test overload

Ruijie(config)#interface cellular 0/0

Ruijie(config-if- cellular 0/0)#ip nat outside

Ruijie(config-if- cellular 0/0)#ip tcp adjust-mss 1200

Ruijie(config)#interface vlan 1

Ruijie(config-if-vlan1)#ip nat outside

Ruijie(config-if-vlan1)#ip tcp adjust-mss 1200

Ruijie(config)#interface vlan xxx         ---//For Lan port

Ruijie(config-if-vlanx)#ip nat inside

Ruijie(config-if-vlanx)#ip tcp adjust-mss 1200